Міністерство освіти і науки України

Львівський національний університет імені Івана Франка

Факультет електроніки та комп'ютерних технологій

ЗВІТ

про виконання лабораторної роботи № 1 
з курсу “Вбудовані системи опрацювання даних та управління на основі нейромереж” 
на тему: “Вивчення лабораторного стенду PSoc 6 DLE Pioneer Kit та середовища ”

Виконав:

Студент. гр. ФеП-41 Фіняк Олег

Перевірив:

Рабик В.Г.

Львів – 2025

Мета роботи: ознайомитися з лабораторним стендом PSoC 6 BLE Pioneer Kit фірми Cypress, основними його компонентами та характеристиками; набути практичних навиків роботи з середовищем розробки PSoC Creator 4.4; реалізувати перший проект на мікроконтролері PSoC 6.

Піни:

Підключення:

Код:
// Code LR_1

#include "project.h"

#define LED_ON  (uint8_t) (0u)

#define LED_OFF (uint8_t) (1u)

#define LED_Port P11_1_PORT

#define LED_Num P11_1_NUM

int main(void)

{

__enable_irq(); /* Enable global interrupts. */

unsigned char n1=5, n2=10, ij;

unsigned int T1=1000, T2=500;

for(;;)

{

for(ij=0; ij<n1; ij++)

{

Cy_GPIO_Write(LED_Port,LED_Num,LED_ON);

CyDelay(T1);

Cy_GPIO_Write(LED_Port,LED_Num,LED_OFF);

CyDelay(T1);

}

for(ij=0; ij<n2; ij++)

{

Cy_GPIO_Write(LED_Port,LED_Num,LED_ON);

CyDelay(T2);

Cy_GPIO_Write(LED_Port,LED_Num,LED_OFF);

CyDelay(T2);

}

}

}

Піни:

Схема:

Код:

#include "project.h"

#define LED_ON   (uint8_t)(0u)

#define LED_OFF  (uint8_t)(1u)

#define LED_RED_PORT    P0_3_PORT

#define LED_RED_NUM     P0_3_NUM

#define LED_BLUE_PORT   P11_1_PORT

#define LED_BLUE_NUM    P11_1_NUM

int main(void)

{

__enable_irq();

unsigned char n1 = 5, ij;

const uint32_t T1 = 750;

const uint32_t T2 = 1250;

for(;;)

{

for (ij = 0; ij<n1; ij++){

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_ON);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_OFF);

CyDelay(T1);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_ON);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_OFF);

CyDelay(T1);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_OFF);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_ON);

CyDelay(T1);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_ON);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_OFF);

CyDelay(T1);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_OFF);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_OFF);

CyDelay(T1);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_ON);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_ON);

CyDelay(T2);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_OFF);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_ON);

CyDelay(T2);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_ON);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_OFF);

CyDelay(T2);

Cy_GPIO_Write(LED_BLUE_PORT, LED_BLUE_NUM, LED_OFF);

Cy_GPIO_Write(LED_RED_PORT, LED_RED_NUM, LED_OFF);

CyDelay(T2);

}

}

}

Висновок: у ході виконання лабораторної роботи я ознайомився з архітектурою та апаратними можливостями налагоджувального стенду PSoC 6 BLE Pioneer Kit, а також опанував інструментарій середовища розробки PSoC Creator 4.4. Я набув практичних навичок створення нових проектів, конфігурування цифрових виводів мікроконтролера (GPIO) та прив’язки їх до фізичних пінів плати. В результаті роботи мною було успішно написано та протестовано програмний код мовою С, який реалізує алгоритми керування RGB-світлодіодом із заданими часовими інтервалами та генерацією різних кольорів.