Міністерство освіти і науки України

Львівський національний університет імені Івана Франка

Факультет електроніки та комп'ютерних технологій

ЗВІТ

про виконання лабораторної роботи № 5

з курсу “Операційні системи реального часу”

на тему: “Таймери програмного рівня та групи подій у FreeRTOS”

Виконав:

Студент. гр. ФеП-41 Фіняк Олег

Перевірив:

Павлик М.Р.

Львів – 2025

Мета роботи. Ознайомитися з принципом роботи таймерів програмного рівня (Software Timers) та груп подій (Event Groups) у FreeRTOS, навчитися використовувати їх для організації подій і синхронізації задач у багатозадачному середовищі на прикладі Raspberry Pi Pico W

/* main.c

FreeRTOS software timer + event group example for Raspberry Pi Pico W

- Timer fires every 1000 ms, toggles LED and sets EVENT_BIT_TIMER_DONE

- TaskA waits for EVENT_BIT_TIMER_DONE and prints tick count

- TaskB simulates a button: when pressed, sets EVENT_BIT_SENSOR_OK and stops timer

- TaskC waits for both bits and performs an action

*/

#include <stdio.h>

#include "pico/stdlib.h"

#include "pico/bootrom.h"

#include "FreeRTOS.h"

#include "task.h"

#include "timers.h"

#include "event_groups.h"

#include "queue.h"

#include "semphr.h"

/* --- Configuration --- */

#define LED_PIN             25      // onboard LED for many Pico boards; change if needed

#define BUTTON_PIN          15      // GPIO for a button (choose an available pin)

#define TIMER_PERIOD_MS     1000U

/* Event bits */

#define EVENT_BIT_TIMER_DONE  (1UL << 0)

#define EVENT_BIT_SENSOR_OK   (1UL << 1)

/* Handles */

static EventGroupHandle_t xEventGroup = NULL;

static TimerHandle_t xBlinkTimer = NULL;

/* Forward declarations */

static void vBlinkTimerCallback(TimerHandle_t xTimer);

static void vTaskA_TimerListener(void *pvParameters);

static void vTaskB_ButtonGenerator(void *pvParameters);

static void vTaskC_WaitBoth(void *pvParameters);

static void gpio_isr(uint gpio, uint32_t events);

/* --- Timer callback: runs in Timer Service Task context --- */

static void vBlinkTimerCallback(TimerHandle_t xTimer)

{

/* Toggle LED (do quick ops only) */

gpio_xor_mask(1u << LED_PIN);

/* Print the current tick count (non-blocking) */

TickType_t ticks = xTaskGetTickCountFromISR();

/* printf is safe-ish here but avoid long blocking operations in timer callback */

// print using low-overhead (note: printf may be thread-safe depending on SDK)

printf("[TimerCallback] tick = %lu\n", (unsigned long)ticks);

/* Set event bit to signal tasks that timer fired.

Use FromISR variant when called from ISR or timer callback context. */

BaseType_t xHigherPriorityTaskWoken = pdFALSE;

xEventGroupSetBitsFromISR(xEventGroup, EVENT_BIT_TIMER_DONE, &xHigherPriorityTaskWoken);

/* If setting the bit woke a higher priority task, request a context switch */

portYIELD_FROM_ISR(xHigherPriorityTaskWoken);

}

/* --- Task A: waits for TIMER_DONE bit and prints message --- */

static void vTaskA_TimerListener(void *pvParameters)

{

(void) pvParameters;

const EventBits_t uxBitsToWaitFor = EVENT_BIT_TIMER_DONE;

for (;;)

{

/* Wait indefinitely for the timer bit. Clear on exit (pdTRUE) so next cycle is fresh */

EventBits_t uxBits = xEventGroupWaitBits(xEventGroup,

uxBitsToWaitFor,

pdTRUE,   // clear on exit

pdFALSE,  // wait for any bit (just this one)

portMAX_DELAY);

if ((uxBits & EVENT_BIT_TIMER_DONE) != 0)

{

TickType_t now = xTaskGetTickCount();

printf("[TaskA] Отримано EVENT_BIT_TIMER_DONE — tick = %lu\n", (unsigned long)now);

}

}

}

/* --- Task B: button monitor / event generator --- */

static void vTaskB_ButtonGenerator(void *pvParameters)

{

(void) pvParameters;

// Polling fallback: also supports ISR-driven event (we attach ISR below).

// We'll implement a simple debounced polling as backup.

const TickType_t xPollDelay = pdMS_TO_TICKS(50);

for (;;)

{

if (gpio_get(BUTTON_PIN) == 0) // assuming active-low button with pull-up

{

// simple debounce

vTaskDelay(pdMS_TO_TICKS(20));

if (gpio_get(BUTTON_PIN) == 0)

{

printf("[TaskB] Button pressed -> setting EVENT_BIT_SENSOR_OK and stopping timer\n");

/* Set event bit from task context */

xEventGroupSetBits(xEventGroup, EVENT_BIT_SENSOR_OK);

/* Stop timer (non-blocking) */

if (xTimerIsTimerActive(xBlinkTimer))

{

xTimerStop(xBlinkTimer, 0);

}

/* Wait until button released to avoid repeated triggers */

while (gpio_get(BUTTON_PIN) == 0)

{

vTaskDelay(pdMS_TO_TICKS(10));

}

}

}

vTaskDelay(xPollDelay);

}

}

/* --- Task C: waits for both bits simultaneously --- */

static void vTaskC_WaitBoth(void *pvParameters)

{

(void) pvParameters;

const EventBits_t uxBitsToWaitFor = (EVENT_BIT_TIMER_DONE | EVENT_BIT_SENSOR_OK);

for (;;)

{

/* Wait for all bits. Clear on exit to allow next round. Timeout optional (e.g., 10s) */

EventBits_t uxBits = xEventGroupWaitBits(xEventGroup,

uxBitsToWaitFor,

pdTRUE,   // clear on exit

pdTRUE,   // wait for all bits

portMAX_DELAY);

if ((uxBits & uxBitsToWaitFor) == uxBitsToWaitFor)

{

TickType_t t = xTaskGetTickCount();

printf("[TaskC] Обидві події (TIMER + SENSOR) отримані на tick=%lu -> виконую дію\n",

(unsigned long)t);

/* Example action: restart timer after handling */

if (!xTimerIsTimerActive(xBlinkTimer))

{

xTimerStart(xBlinkTimer, 0);

printf("[TaskC] Таймер перезапущено.\n");

}

}

}

}

/* --- Optional GPIO ISR (fast) to set sensor bit from ISR context --- */

static void gpio_isr(uint gpio, uint32_t events)

{

(void) events;

if (gpio == BUTTON_PIN)

{

BaseType_t xHigherPriorityTaskWoken = pdFALSE;

/* Set the bit from ISR */

xEventGroupSetBitsFromISR(xEventGroup, EVENT_BIT_SENSOR_OK, &xHigherPriorityTaskWoken);

/* Stop timer from ISR: use a deferred command via timer/queue or set a flag.

Here we request a context switch only. (Stopping timers from ISR is not recommended directly.) */

portYIELD_FROM_ISR(xHigherPriorityTaskWoken);

}

}

/* --- Main --- */

int main(void)

{

stdio_init_all();

printf("FreeRTOS Timers + EventGroups demo starting...\n");

/* Initialize GPIOs */

gpio_init(LED_PIN);

gpio_set_dir(LED_PIN, GPIO_OUT);

gpio_put(LED_PIN, 0);

// Button: enable pull-up, input

gpio_init(BUTTON_PIN);

gpio_set_dir(BUTTON_PIN, GPIO_IN);

gpio_pull_up(BUTTON_PIN);

/* Create Event Group */

xEventGroup = xEventGroupCreate();

if (xEventGroup == NULL)

{

printf("Failed to create event group\n");

while (1) tight_loop_contents();

}

/* Create a periodic software timer (auto-reload) that calls vBlinkTimerCallback every TIMER_PERIOD_MS */

xBlinkTimer = xTimerCreate("BlinkTimer",

pdMS_TO_TICKS(TIMER_PERIOD_MS),

pdTRUE,    // pdTRUE => auto-reload

(void *)0,

vBlinkTimerCallback);

if (xBlinkTimer == NULL)

{

printf("Failed to create timer\n");

while (1) tight_loop_contents();

}

/* Start the timer */

if (xTimerStart(xBlinkTimer, 0) != pdPASS)

{

printf("Failed to start timer\n");

}

/* Create tasks */

BaseType_t ret;

ret = xTaskCreate(vTaskA_TimerListener, "TaskA", 512, NULL, tskIDLE_PRIORITY + 2, NULL);

if (ret != pdPASS) { printf("TaskA creation failed\n"); }

ret = xTaskCreate(vTaskB_ButtonGenerator, "TaskB", 512, NULL, tskIDLE_PRIORITY + 1, NULL);

if (ret != pdPASS) { printf("TaskB creation failed\n"); }

ret = xTaskCreate(vTaskC_WaitBoth, "TaskC", 512, NULL, tskIDLE_PRIORITY + 3, NULL);

if (ret != pdPASS) { printf("TaskC creation failed\n"); }

/* Optionally attach ISR on falling edge (active low button) */

// Note: attaching ISR requires that ISR installs are supported; also we used polling in TaskB as backup

// gpio_set_irq_enabled_with_callback(BUTTON_PIN, GPIO_IRQ_EDGE_FALL, true, &gpio_isr);

/* Start the scheduler */

vTaskStartScheduler();

/* Should never reach here */

for (;;)

{

tight_loop_contents();

}

return 0;

}

Висновок: лабораторна демонструє, як програмні таймери FreeRTOS і групи подій дозволяють ефективно синхронізувати поведінку задач без створення додаткових задач для простих періодичних подій. Таймер виконує швидку дію (переключення LED і встановлення біту), а задачі реагують на біти подій, реалізуючи тривалі або блокуючі операції. Це розділення обробки (швидкий таймер / deferred tasks) робить систему більш передбачуваною та безпечною для RTOS-середовища. Реалізація на Raspberry Pi Pico W потребує уважності до контекстів виконання (ISR vs task) та коректного використання FromISR API.