Міністерство освіти і науки України

Львівський національний університет імені Івана Франка

Факультет електроніки та комп'ютерних технологій

ЗВІТ

про виконання лабораторної роботи № 6

з курсу “Операційні системи реального часу”

на тему: “Обробка аналогових даних у багатозадачній системі”

Виконав:

Студент. гр. ФеП-41 Фіняк Олег

Перевірив:

Павлик М.Р.

Львів – 2025

Мета роботи. Реалізувати багатозадачну систему, у якій одна задача зчитує аналогові дані з потенціометра або фоторезистора, а інша – відображає їх на OLED-дисплеї.

#include <stdio.h>

#include <stdlib.h>

#include "pico/stdlib.h"

#include "hardware/adc.h"

#include "hardware/gpio.h"

#include "FreeRTOS.h"

#include "task.h"

#include "queue.h"

#include "DEV_Config.h"

#include "OLED_1in5.h"

#include "GUI_Paint.h"

#include "Fonts.h"

#define BUTTON_PIN 15

#define POT_PIN 26

#define LIGHT_PIN 27

#define SOUND_PIN 28

volatile bool display_on = true;

QueueHandle_t xQueueButton;

QueueHandle_t xQueuePot;

QueueHandle_t xQueueLight;

QueueHandle_t xQueueSound;

UBYTE *BlackImage;

/* =================== Ініціалізація ADC =================== */

void init_adc() {

adc_init();

adc_gpio_init(POT_PIN);

adc_gpio_init(LIGHT_PIN);

adc_gpio_init(SOUND_PIN);

}

/* =================== Задача кнопки =================== */

void vButtonTask(void *pvParameters) {

bool last_state = false;

bool current_state;

while (1) {

current_state = !gpio_get(BUTTON_PIN);

if (current_state != last_state) {

vTaskDelay(pdMS_TO_TICKS(20));  // debounce

current_state = !gpio_get(BUTTON_PIN);

if (current_state != last_state) {

xQueueSend(xQueueButton, &current_state, 0);

last_state = current_state;

}

}

vTaskDelay(pdMS_TO_TICKS(50));

}

}

/* =================== Задача читання ADC =================== */

void vSensorTask(void *pvParameters) {

uint8_t channel = (uint8_t)(uintptr_t) pvParameters;

uint16_t value;

while (1) {

adc_select_input(channel);

value = adc_read();

if (channel == 0) xQueueOverwrite(xQueuePot, &value);

else if (channel == 1) xQueueOverwrite(xQueueLight, &value);

else if (channel == 2) xQueueOverwrite(xQueueSound, &value);

vTaskDelay(pdMS_TO_TICKS(100));

}

}

/* =================== Прогрес бар =================== */

void draw_progress_bar(int x, int y, int width, int height, uint8_t percent) {

int fill = (width * percent) / 100;

Paint_DrawRectangle(x, y, x + width, y + height, WHITE, DOT_PIXEL_1X1, DRAW_FILL_EMPTY);

Paint_DrawRectangle(x+1, y+1, x + fill, y + height - 1, WHITE, DOT_PIXEL_1X1, DRAW_FILL_FULL);

}

/* =================== Задача дисплея =================== */

void vDisplayTask(void *pvParameters) {

uint16_t pot = 0, light = 0, sound = 0;

bool btn;

while (1) {

if (xQueueReceive(xQueueButton, &btn, 0)) {

if (btn) display_on = !display_on;

}

if (!display_on) {

OLED_1in5_Clear(0);

vTaskDelay(pdMS_TO_TICKS(200));

continue;

}

xQueueReceive(xQueuePot, &pot, 0);

xQueueReceive(xQueueLight, &light, 0);

xQueueReceive(xQueueSound, &sound, 0);

uint8_t pot_percent = (pot * 100) / 4095;

uint8_t light_percent = (light * 100) / 4095;

uint8_t sound_percent = (sound * 100) / 4095;

Paint_Clear(BLACK);

Paint_DrawString_EN(5, 0, "Potentiometer", &Font12, WHITE, BLACK);

draw_progress_bar(5, 15, 110, 10, pot_percent);

Paint_DrawString_EN(5, 35, "Light Sensor", &Font12, WHITE, BLACK);

draw_progress_bar(5, 50, 110, 10, light_percent);

Paint_DrawString_EN(5, 70, "Sound Sensor", &Font12, WHITE, BLACK);

draw_progress_bar(5, 85, 110, 10, sound_percent);

OLED_1in5_Display(BlackImage);

vTaskDelay(pdMS_TO_TICKS(250));

}

}

/* =================== Головна функція =================== */

int main() {

stdio_init_all();

gpio_init(BUTTON_PIN);

gpio_set_dir(BUTTON_PIN, GPIO_IN);

gpio_pull_up(BUTTON_PIN);

init_adc();

if (DEV_Module_Init() != 0) {

while (1);

}

OLED_1in5_Init();

OLED_1in5_Clear(0);

UWORD Imagesize = ((OLED_1in5_WIDTH % 2 == 0) ? (OLED_1in5_WIDTH / 2) :

(OLED_1in5_WIDTH / 2 + 1)) * OLED_1in5_HEIGHT;

BlackImage = (UBYTE *)malloc(Imagesize);

Paint_NewImage(BlackImage, OLED_1in5_WIDTH, OLED_1in5_HEIGHT, 0, WHITE);

Paint_SetScale(16);

Paint_SelectImage(BlackImage);

xQueueButton = xQueueCreate(5, sizeof(bool));

xQueuePot = xQueueCreate(1, sizeof(uint16_t));

xQueueLight = xQueueCreate(1, sizeof(uint16_t));

xQueueSound = xQueueCreate(1, sizeof(uint16_t));

xTaskCreate(vButtonTask, "ButtonTask", 256, NULL, 1, NULL);

xTaskCreate(vSensorTask, "PotTask", 256, (void*)0, 1, NULL);

xTaskCreate(vSensorTask, "LightTask", 256, (void*)1, 1, NULL);

xTaskCreate(vSensorTask, "SoundTask", 256, (void*)2, 1, NULL);

xTaskCreate(vDisplayTask, "DisplayTask", 512, NULL, 2, NULL);

vTaskStartScheduler();

while (1);

}

Висновок: у ході лабораторної роботи було реалізовано багатозадачну систему на базі FreeRTOS із використанням ADC, черг та OLED-дисплея. Було практично освоєно механізми міжзадачної взаємодії за допомогою черг та керування периферією RP2040. Реалізований інтерфейс з прогрес-барами дозволив наочно відображати аналогові сигнали сенсорів у реальному часі. Отримані навички є ключовими для проєктування вбудованих систем моніторингу та керування.