Міністерство освіти і науки України

Львівський національний університет імені Івана Франка

Факультет електроніки та комп'ютерних технологій

ЗВІТ

про виконання лабораторної роботи № 10

з курсу “Операційні системи реального часу”

на тему: “Розробка багатозадачної інтерактивної гри для міні-консолі”

Виконав:

Студент. гр. ФеП-41 Фіняк Олег

Перевірив:

Павлик М.Р.

Львів – 2025

Мета роботи. Навчитися розробляти прості інтерактивні ігри типу “пінг-понг” та “змійка” для міні-консолі з використанням механізмів RTOS.

/**

* @file lab-10.c

* @brief FreeRTOS-based multi-game system for Raspberry Pi Pico with LCD display

*

* This program implements a real-time multi-game system using FreeRTOS on a Raspberry Pi Pico.

* It consists of multiple tasks demonstrating:

* - FreeRTOS Tasks: Separate tasks for input handling, game logic, and display rendering

* - FreeRTOS Queues: Message passing between tasks for input events

* - FreeRTOS Semaphores: Synchronization for frame buffer access and display updates

* - LCD Display: Real-time rendering of game graphics on 1.3 inch LCD screen

* - Multiple Games: Snake game, Math Quiz game, and menu system

*

* The system uses FreeRTOS queues to decouple input reading from game logic, and semaphores

* to protect shared frame buffer access. Each game runs in a dedicated game logic task that

* processes input events and updates the display. The display task handles LCD refresh timing.

*

* @author Mykyta Siryk

* @date 2025-12-03

* @version 1.0

*/

// Standard C library for input/output operations and formatted string functions

#include <stdio.h>

// Standard C library for memory allocation functions (malloc, free)

#include <stdlib.h>

// Standard C library for string manipulation functions (memcmp, etc.)

#include <string.h>

// Raspberry Pi Pico SDK for hardware access (GPIO, timing, etc.)

#include "pico/stdlib.h"

// FreeRTOS core header - provides task management, scheduling, etc.

#include "FreeRTOS.h"

// FreeRTOS task management functions (xTaskCreate, vTaskDelay, etc.)

#include "task.h"

// FreeRTOS queue management functions (xQueueCreate, xQueueSend, xQueueReceive, etc.)

#include "queue.h"

// FreeRTOS semaphore and mutex management functions (xSemaphoreCreate, etc.)

#include "semphr.h"

// Waveshare LCD display driver - device configuration and initialization

#include "DEV_Config.h"

// Waveshare LCD display driver - graphics and drawing functions

#include "GUI_Paint.h"

// Waveshare LCD display driver - 1.3 inch LCD display controller

#include "LCD_1in3.h"

/* ================== GPIO Pin Definitions ================== */

/** @brief GPIO pin number for button A input */

#define BTN_A 15

/** @brief GPIO pin number for button B input */

#define BTN_B 17

/** @brief GPIO pin number for directional button UP input */

#define BTN_UP 2

/** @brief GPIO pin number for directional button DOWN input */

#define BTN_DOWN 18

/** @brief GPIO pin number for directional button LEFT input */

#define BTN_LEFT 16

/** @brief GPIO pin number for directional button RIGHT input */

#define BTN_RIGHT 20

/** @brief GPIO pin number for control button input */

#define BTN_CTRL 3

/* ================== Timing Constants ================== */

/** @brief Button debounce delay time in milliseconds */

#define DEBOUNCE_TIME 50

/* ================== Color Definitions (RGB565 format) ================== */

/** @brief Background color (white) */

#define COLOR_BG WHITE

/** @brief Foreground color (black) */

#define COLOR_FG BLACK

/** @brief Obstacle/enemy color (red) */

#define COLOR_OBSTACLE 0xF800

/** @brief Snake color (blue) */

#define COLOR_SNAKE 0x001F

/** @brief Food color (red) */

#define COLOR_FOOD 0xF800

/* ================== Data Structures ================== */

/**

* @brief Structure representing the current state of all input buttons

*

* This structure stores the pressed/released state of all game control buttons.

* Each boolean field indicates whether the corresponding button is currently pressed.

*/

typedef struct {

bool btnA, btnB, btnUp, btnDown, btnLeft, btnRight, btnCtrl;

} InputState;

/**

* @brief Enumeration of available game types

*

* Defines the different game modes available in the system:

* - GAME_MENU: Main menu screen

* - GAME_MATH_QUIZ: Math quiz game

* - GAME_SNAKE: Snake game

*/

typedef enum { GAME_MENU, GAME_MATH_QUIZ, GAME_SNAKE } GameType;

/* ================== Global Variables ================== */

/** @brief Pointer to the LCD frame buffer for drawing operations */

UWORD *frameBuffer;

/** @brief FreeRTOS queue handle for input state messages from input task to game logic task */

QueueHandle_t inputQueue;

/** @brief FreeRTOS binary semaphore handle for signaling when a new frame is ready to display */

SemaphoreHandle_t frameSemaphore;

/** @brief FreeRTOS mutex handle for protecting frame buffer access from concurrent tasks */

SemaphoreHandle_t bufferMutex;

/** @brief Current active game type/mode */

GameType currentGame = GAME_MENU;

/** @brief High score for Snake game */

int snakeHighScore = 0;

/* ================== SNAKE GAME ================== */

/** @brief Maximum length of the snake (maximum number of segments) */

#define SNAKE_MAX 100

/** @brief Size of each grid cell in pixels */

#define GRID_SIZE 12

/** @brief Width of the game grid in cells */

#define GRID_W 20

/** @brief Height of the game grid in cells */

#define GRID_H 20

/**

* @brief Structure representing the complete state of the Snake game

*

* This structure stores all game state including snake position, direction,

* food location, score, and game over status.

*/

typedef struct {

int x[SNAKE_MAX], y[SNAKE_MAX];  /**< Array of x,y coordinates for each snake segment */

int len;                          /**< Current length of the snake (number of segments) */

int dir;                          /**< Current direction (0=right, 1=down, 2=left, 3=up) */

int foodX, foodY;                 /**< Grid coordinates of the food */

int score;                        /**< Current game score */

bool gameOver;                    /**< Flag indicating if the game is over */

} SnakeGame;

/** @brief Global instance of the Snake game state */

SnakeGame snake;

/**

* @brief Spawn food at a random location that doesn't overlap with the snake

*

* This function randomly places food on the game grid, ensuring it doesn't

* spawn on any position currently occupied by the snake's body segments.

* The function uses a validation loop to find a valid empty position.

*

* @note The food position is stored in snake.foodX and snake.foodY

* @note Uses rand() for random number generation

*/

void spawnFood() {

bool valid;

// Keep trying random positions until we find one that doesn't overlap with snake

do {

valid = true;

// Generate random grid coordinates

snake.foodX = rand() % GRID_W;

snake.foodY = rand() % GRID_H;

// Check if the random position overlaps with any snake segment

for (int i = 0; i < snake.len; i++) {

if (snake.x[i] == snake.foodX && snake.y[i] == snake.foodY) {

valid = false;

break;

}

}

} while (!valid);

}

/**

* @brief Initialize the Snake game to starting state

*

* This function resets the snake game to its initial state:

* - Sets snake length to 3 segments

* - Positions snake horizontally at (5,5), (4,5), (3,5)

* - Sets initial direction to right (0)

* - Resets score to 0

* - Clears game over flag

* - Spawns initial food

*/

void initSnake() {

// Initialize snake with 3 segments

snake.len = 3;

// Set initial direction to right (0 = right, 1 = down, 2 = left, 3 = up)

snake.dir = 0;

// Position snake horizontally: head at (5,5), body segments at (4,5) and (3,5)

snake.x[0] = 5; snake.y[0] = 5;

snake.x[1] = 4; snake.y[1] = 5;

snake.x[2] = 3; snake.y[2] = 5;

// Reset game state

snake.score = 0;

snake.gameOver = false;

// Spawn initial food at a random valid location

spawnFood();

}

/**

* @brief Update Snake game state based on input and game logic

*

* This function processes one game tick of the Snake game:

* - Handles game over state (restart on button press)

* - Processes directional input (prevents 180-degree turns)

* - Moves snake by shifting body segments and updating head position

* - Checks for wall collisions

* - Checks for self-collision

* - Handles food consumption (grow snake, increase score, spawn new food)

*

* @param input Pointer to InputState structure containing current button states

*

* @note Direction values: 0=right, 1=down, 2=left, 3=up

* @note Prevents reversing direction (e.g., can't go up if currently going down)

*/

void updateSnake(InputState *input) {

// If game is over, check for restart input

if (snake.gameOver) {

if (input->btnA || input->btnUp) initSnake();

return;

}

// Process directional input (prevent 180-degree turns)

// Only change direction if not opposite to current direction

if (input->btnUp && snake.dir != 1) snake.dir = 3;      // Up (if not going down)

if (input->btnDown && snake.dir != 3) snake.dir = 1;    // Down (if not going up)

if (input->btnLeft && snake.dir != 0) snake.dir = 2;   // Left (if not going right)

if (input->btnRight && snake.dir != 2) snake.dir = 0;  // Right (if not going left)

// Move snake body: shift each segment to position of previous segment

// Start from tail and work forward to avoid overwriting positions

for (int i = snake.len - 1; i > 0; i--) {

snake.x[i] = snake.x[i-1];

snake.y[i] = snake.y[i-1];

}

// Move snake head based on current direction

if (snake.dir == 0) snake.x[0]++;  // Right

if (snake.dir == 1) snake.y[0]++;  // Down

if (snake.dir == 2) snake.x[0]--;  // Left

if (snake.dir == 3) snake.y[0]--;  // Up

// Check for wall collision (head out of bounds)

if (snake.x[0] < 0 || snake.x[0] >= GRID_W || snake.y[0] < 0 || snake.y[0] >= GRID_H) {

snake.gameOver = true;

return;

}

// Check for self-collision (head overlaps with any body segment)

for (int i = 1; i < snake.len; i++) {

if (snake.x[0] == snake.x[i] && snake.y[0] == snake.y[i]) {

snake.gameOver = true;

return;

}

}

// Check if snake head reached food

if (snake.x[0] == snake.foodX && snake.y[0] == snake.foodY) {

// Grow snake if not at maximum length

if (snake.len < SNAKE_MAX) {

snake.len++;

}

// Increase score

snake.score += 10;

// Update high score if current score is higher

if (snake.score > snakeHighScore) snakeHighScore = snake.score;

// Spawn new food at random location

spawnFood();

}

}

/**

* @brief Render the Snake game to the frame buffer

*

* This function draws the complete Snake game screen including:

* - Game grid border

* - Snake body segments (head in brighter blue, body in darker blue)

* - Food item (red circle)

* - Score and high score display

* - Game over overlay (if game is over)

*

* The function uses mutex protection to safely access the frame buffer,

* then signals the display task that a new frame is ready.

*

* @note Takes bufferMutex before drawing and gives frameSemaphore after drawing

* @note Head has eyes drawn on it for visual distinction

*/

void drawSnake() {

// Take mutex to protect frame buffer access

xSemaphoreTake(bufferMutex, portMAX_DELAY);

// Clear screen to background color

Paint_Clear(COLOR_BG);

// Draw game grid border

Paint_DrawRectangle(0, 0, GRID_W * GRID_SIZE, GRID_H * GRID_SIZE,

COLOR_FG, DOT_PIXEL_2X2, DRAW_FILL_EMPTY);

// Draw each snake segment

for (int i = 0; i < snake.len; i++) {

// Convert grid coordinates to pixel coordinates (with 2px padding)

int px = snake.x[i] * GRID_SIZE + 2;

int py = snake.y[i] * GRID_SIZE + 2;

// Head is brighter blue, body is darker blue

UWORD color = (i == 0) ? 0x001F : 0x0014;

// Draw snake segment as filled rectangle

Paint_DrawRectangle(px, py, px + GRID_SIZE - 4, py + GRID_SIZE - 4,

color, DOT_PIXEL_1X1, DRAW_FILL_FULL);

// Draw eyes on snake head

if (i == 0) {

Paint_DrawRectangle(px + 2, py + 2, px + 3, py + 3,

COLOR_BG, DOT_PIXEL_1X1, DRAW_FILL_FULL);

Paint_DrawRectangle(px + 6, py + 2, px + 7, py + 3,

COLOR_BG, DOT_PIXEL_1X1, DRAW_FILL_FULL);

}

}

// Draw food as a red circle

int fx = snake.foodX * GRID_SIZE + 2;

int fy = snake.foodY * GRID_SIZE + 2;

Paint_DrawCircle(fx + GRID_SIZE/2 - 2, fy + GRID_SIZE/2 - 2,

4, COLOR_FOOD, DOT_PIXEL_1X1, DRAW_FILL_FULL);

// Display score and high score at bottom of screen

char txt[40];

sprintf(txt, "Score:%d Best:%d", snake.score, snakeHighScore);

Paint_DrawString_EN(10, 245, txt, &Font12, COLOR_FG, COLOR_BG);

// Draw game over overlay if game is over

if (snake.gameOver) {

// Draw semi-transparent overlay background

Paint_DrawRectangle(40, 90, 200, 150, COLOR_BG, DOT_PIXEL_1X1, DRAW_FILL_FULL);

// Draw overlay border

Paint_DrawRectangle(40, 90, 200, 150, COLOR_FG, DOT_PIXEL_2X2, DRAW_FILL_EMPTY);

// Draw "GAME OVER" text

Paint_DrawString_EN(70, 105, "GAME OVER", &Font16, COLOR_OBSTACLE, COLOR_BG);

// Draw final score

sprintf(txt, "Score: %d", snake.score);

Paint_DrawString_EN(80, 125, txt, &Font12, COLOR_FG, COLOR_BG);

}

// Release mutex and signal display task that frame is ready

xSemaphoreGive(bufferMutex);

xSemaphoreGive(frameSemaphore);

}

/* ================== MATH QUIZ GAME ================== */

/** @brief Number of questions in the math quiz */

#define MATH_QUIZ_QUESTIONS 3

/**

* @brief Enumeration of math quiz game states

*

* Defines the different states the math quiz can be in:

* - MATH_STATE_PLAYING: Showing question and waiting for answer

* - MATH_STATE_SHOWING_RESULT: Showing feedback after answering

* - MATH_STATE_COMPLETE: Quiz finished, showing final score

*/

typedef enum {

MATH_STATE_PLAYING,

MATH_STATE_SHOWING_RESULT,

MATH_STATE_COMPLETE

} MathQuizState;

/**

* @brief Structure representing a single math question

*

* Contains the question data, correct answer, wrong answers, and

* the shuffled answer array for display.

*/

typedef struct {

int operand1;              /**< First operand in the math problem */

int operand2;              /**< Second operand in the math problem */

char operation;            /**< Operation character ('+' or '-') */

int correctAnswer;         /**< The correct answer to the problem */

int wrongAnswer1;          /**< First wrong answer option */

int wrongAnswer2;          /**< Second wrong answer option */

int correctIndex;          /**< Which position (0-2) has the correct answer in answers[] */

int answers[3];            /**< Shuffled answers array for display (correct + 2 wrong) */

} MathQuestion;

/**

* @brief Structure representing the complete state of the Math Quiz game

*

* Contains current question index, selected answer choice, score,

* game state, all questions, and feedback timing.

*/

typedef struct {

int currentQuestion;                        /**< Index of current question (0 to MATH_QUIZ_QUESTIONS-1) */

int selectedChoice;                         /**< Currently selected answer choice (0-2) */

int score;                                  /**< Number of correct answers */

MathQuizState state;                        /**< Current game state */

MathQuestion questions[MATH_QUIZ_QUESTIONS]; /**< Array of all quiz questions */

bool showingFeedback;                       /**< Flag indicating if feedback is being shown */

TickType_t feedbackStartTime;              /**< Tick count when feedback started (for timing) */

} MathQuizGame;

/** @brief Global instance of the Math Quiz game state */

MathQuizGame mathQuiz;

/**

* @brief Initialize the math quiz questions with fixed problems

*

* This function sets up three predefined math questions with double-digit numbers.

* Each question includes the operands, operation, correct answer, and two wrong

* answers. The answers are shuffled and stored in the answers[] array with the

* correctIndex indicating which position contains the correct answer.

*

* Questions:

* 1. 45 + 32 = 77 (correct at position 1)

* 2. 78 - 24 = 54 (correct at position 0)

* 3. 56 + 19 = 75 (correct at position 2)

*/

void initMathQuestions() {

// Question 1: 45 + 32 = 77

mathQuiz.questions[0].operand1 = 45;

mathQuiz.questions[0].operand2 = 32;

mathQuiz.questions[0].operation = '+';

mathQuiz.questions[0].correctAnswer = 77;

mathQuiz.questions[0].wrongAnswer1 = 68;

mathQuiz.questions[0].wrongAnswer2 = 85;

// Shuffle answers: correct at position 1

mathQuiz.questions[0].answers[0] = 68;

mathQuiz.questions[0].answers[1] = 77; // correct

mathQuiz.questions[0].answers[2] = 85;

mathQuiz.questions[0].correctIndex = 1;

// Question 2: 78 - 24 = 54

mathQuiz.questions[1].operand1 = 78;

mathQuiz.questions[1].operand2 = 24;

mathQuiz.questions[1].operation = '-';

mathQuiz.questions[1].correctAnswer = 54;

mathQuiz.questions[1].wrongAnswer1 = 52;

mathQuiz.questions[1].wrongAnswer2 = 56;

// Shuffle answers: correct at position 0

mathQuiz.questions[1].answers[0] = 54; // correct

mathQuiz.questions[1].answers[1] = 52;

mathQuiz.questions[1].answers[2] = 56;

mathQuiz.questions[1].correctIndex = 0;

// Question 3: 56 + 19 = 75

mathQuiz.questions[2].operand1 = 56;

mathQuiz.questions[2].operand2 = 19;

mathQuiz.questions[2].operation = '+';

mathQuiz.questions[2].correctAnswer = 75;

mathQuiz.questions[2].wrongAnswer1 = 73;

mathQuiz.questions[2].wrongAnswer2 = 77;

// Shuffle answers: correct at position 2

mathQuiz.questions[2].answers[0] = 73;

mathQuiz.questions[2].answers[1] = 77;

mathQuiz.questions[2].answers[2] = 75; // correct

mathQuiz.questions[2].correctIndex = 2;

}

/**

* @brief Initialize the Math Quiz game to starting state

*

* This function resets the math quiz to its initial state:

* - Sets current question to first question (index 0)

* - Resets selected choice to first option (0)

* - Resets score to 0

* - Sets state to playing

* - Clears feedback flag

* - Initializes all questions

*/

void initMathQuiz() {

// Reset to first question

mathQuiz.currentQuestion = 0;

// Reset selected choice to first option

mathQuiz.selectedChoice = 0;

// Reset score

mathQuiz.score = 0;

// Set initial state to playing

mathQuiz.state = MATH_STATE_PLAYING;

// Clear feedback flag

mathQuiz.showingFeedback = false;

// Initialize all questions

initMathQuestions();

}

/**

* @brief Update Math Quiz game state based on input and timing

*

* This function processes one game tick of the Math Quiz:

* - Handles quiz complete state (return to menu on button press)

* - Manages feedback timing (2 second delay before next question)

* - Processes navigation input (LEFT/RIGHT to select answer choice)

* - Handles answer selection (Button A to submit answer)

* - Updates score based on correct/incorrect answer

* - Advances to next question or completes quiz

*

* @param input Pointer to InputState structure containing current button states

*

* @note Feedback is shown for 2 seconds before advancing

* @note Quiz completes after all questions are answered

*/

void updateMathQuiz(InputState *input) {

// If quiz is complete, allow return to menu

if (mathQuiz.state == MATH_STATE_COMPLETE) {

if (input->btnA) {

currentGame = GAME_MENU;

}

return;

}

// If showing feedback, wait for timeout before advancing

if (mathQuiz.showingFeedback) {

// Wait 2 seconds before moving to next question

if (xTaskGetTickCount() - mathQuiz.feedbackStartTime >= pdMS_TO_TICKS(2000)) {

mathQuiz.showingFeedback = false;

// Advance to next question

mathQuiz.currentQuestion++;

// Check if all questions completed

if (mathQuiz.currentQuestion >= MATH_QUIZ_QUESTIONS) {

mathQuiz.state = MATH_STATE_COMPLETE;

} else {

// Reset for next question

mathQuiz.selectedChoice = 0;

mathQuiz.state = MATH_STATE_PLAYING;

}

}

return;

}

// Process input during playing state

if (mathQuiz.state == MATH_STATE_PLAYING) {

// Navigate between answer choices with LEFT/RIGHT buttons

if (input->btnLeft && mathQuiz.selectedChoice > 0) {

mathQuiz.selectedChoice--;

}

if (input->btnRight && mathQuiz.selectedChoice < 2) {

mathQuiz.selectedChoice++;

}

// Select answer with Button A

if (input->btnA) {

MathQuestion *q = &mathQuiz.questions[mathQuiz.currentQuestion];

int selectedAnswer = q->answers[mathQuiz.selectedChoice];

// Check if selected answer is correct

if (mathQuiz.selectedChoice == q->correctIndex) {

mathQuiz.score++;

}

// Start showing feedback

mathQuiz.showingFeedback = true;

mathQuiz.feedbackStartTime = xTaskGetTickCount();

mathQuiz.state = MATH_STATE_SHOWING_RESULT;

}

}

}

/**

* @brief Render the Math Quiz game to the frame buffer

*

* This function draws the math quiz screen based on current game state:

* - Playing state: Shows question, three answer choices (with selection highlight)

* - Feedback state: Shows question, selected answer, and correct/incorrect feedback

* - Complete state: Shows final score and return instructions

*

* The function uses mutex protection to safely access the frame buffer,

* then signals the display task that a new frame is ready.

*

* @note Takes bufferMutex before drawing and gives frameSemaphore after drawing

* @note Selected answer choice is highlighted with red background

*/

void drawMathQuiz() {

// Take mutex to protect frame buffer access

xSemaphoreTake(bufferMutex, portMAX_DELAY);

// Clear screen to background color

Paint_Clear(COLOR_BG);

if (mathQuiz.state == MATH_STATE_COMPLETE) {

// Final score screen - quiz is complete

Paint_DrawString_EN(50, 50, "Quiz Complete!", &Font20, COLOR_FG, COLOR_BG);

char txt[40];

sprintf(txt, "Score: %d/%d", mathQuiz.score, MATH_QUIZ_QUESTIONS);

Paint_DrawString_EN(70, 100, txt, &Font20, COLOR_FG, COLOR_BG);

// Instructions to return to menu

Paint_DrawString_EN(40, 180, "Press A to return", &Font12, COLOR_FG, COLOR_BG);

Paint_DrawString_EN(40, 200, "B - Back to Menu", &Font12, COLOR_FG, COLOR_BG);

} else if (mathQuiz.showingFeedback) {

// Show feedback after answering

MathQuestion *q = &mathQuiz.questions[mathQuiz.currentQuestion];

bool isCorrect = (mathQuiz.selectedChoice == q->correctIndex);

char txt[60];

// Show question number

sprintf(txt, "Question %d/%d", mathQuiz.currentQuestion + 1, MATH_QUIZ_QUESTIONS);

Paint_DrawString_EN(50, 30, txt, &Font16, COLOR_FG, COLOR_BG);

// Show the math problem

sprintf(txt, "%d %c %d = ?", q->operand1, q->operation, q->operand2);

Paint_DrawString_EN(60, 80, txt, &Font20, COLOR_FG, COLOR_BG);

// Show feedback based on correctness

if (isCorrect) {

Paint_DrawString_EN(80, 130, "Correct!", &Font20, COLOR_OBSTACLE, COLOR_BG);

} else {

Paint_DrawString_EN(70, 130, "Incorrect!", &Font20, COLOR_OBSTACLE, COLOR_BG);

sprintf(txt, "Answer: %d", q->correctAnswer);

Paint_DrawString_EN(70, 160, txt, &Font16, COLOR_FG, COLOR_BG);

}

} else {

// Show question and answer choices (playing state)

MathQuestion *q = &mathQuiz.questions[mathQuiz.currentQuestion];

char txt[60];

// Show question number

sprintf(txt, "Question %d/%d", mathQuiz.currentQuestion + 1, MATH_QUIZ_QUESTIONS);

Paint_DrawString_EN(50, 20, txt, &Font16, COLOR_FG, COLOR_BG);

// Show the math problem

sprintf(txt, "%d %c %d = ?", q->operand1, q->operation, q->operand2);

Paint_DrawString_EN(60, 60, txt, &Font20, COLOR_FG, COLOR_BG);

// Show three answer choices

int yStart = 120;

int ySpacing = 35;

for (int i = 0; i < 3; i++) {

int y = yStart + i * ySpacing;

UWORD bgColor = COLOR_BG;

UWORD textColor = COLOR_FG;

// Highlight selected choice with red background

if (i == mathQuiz.selectedChoice) {

bgColor = COLOR_OBSTACLE;

textColor = COLOR_BG;

// Draw background rectangle for selected choice

Paint_DrawRectangle(70, y - 2, 170, y + 18, bgColor, DOT_PIXEL_1X1, DRAW_FILL_FULL);

}

// Draw answer choice label and value

sprintf(txt, "%c) %d", 'A' + i, q->answers[i]);

Paint_DrawString_EN(80, y, txt, &Font16, textColor, bgColor);

}

// Display control instructions

Paint_DrawString_EN(30, 230, "LEFT/RIGHT - Select  A - Answer", &Font12, COLOR_FG, COLOR_BG);

}

// Release mutex and signal display task that frame is ready

xSemaphoreGive(bufferMutex);

xSemaphoreGive(frameSemaphore);

}

/* ================== MENU SYSTEM ================== */

/** @brief Currently selected menu item index (0 = Math Quiz, 1 = Snake, 2 = Power Off) */

int menuSelection = 0;

/**

* @brief Update menu state based on input

*

* This function processes menu navigation and selection:

* - UP/DOWN buttons navigate between menu items (0-2)

* - Button A or CTRL selects the current menu item

* - Menu item 0: Start Math Quiz game

* - Menu item 1: Start Snake game

* - Menu item 2: Power off (clear screen, disable backlight, enter infinite loop)

*

* @param input Pointer to InputState structure containing current button states

*

* @note Power off option clears the screen, disables backlight, and enters infinite loop

* @note Menu selection range is 0-2 (3 options)

*/

void updateMenu(InputState *input) {

// Navigate menu with UP/DOWN buttons (range 0-2)

if (input->btnUp && menuSelection > 0) menuSelection--;

if (input->btnDown && menuSelection < 2) menuSelection++;

// Handle menu selection with Button A or CTRL

if (input->btnA || input->btnCtrl) {

if (menuSelection == 0) {

// Start Math Quiz game

currentGame = GAME_MATH_QUIZ;

initMathQuiz();

} else if (menuSelection == 1) {

// Start Snake game

currentGame = GAME_SNAKE;

initSnake();

} else if (menuSelection == 2) {

// Power off: clear screen, disable backlight, stop execution

xSemaphoreTake(bufferMutex, portMAX_DELAY);

Paint_Clear(BLACK); // Clear screen to black

LCD_1IN3_Display(frameBuffer); // Display black screen

DEV_SET_PWM(0); // Disable backlight

xSemaphoreGive(bufferMutex);

// Enter infinite loop to stop all logic

while(1) {

vTaskDelay(pdMS_TO_TICKS(1000));

}

}

}

}

/**

* @brief Render the menu screen to the frame buffer

*

* This function draws the main menu screen including:

* - Menu title "GAME MENU"

* - Three menu options (Math Quiz, Snake, Power Off)

* - Highlighted selection (selected item shown in red)

* - Control instructions at bottom

*

* The function uses mutex protection to safely access the frame buffer,

* then signals the display task that a new frame is ready.

*

* @note Takes bufferMutex before drawing and gives frameSemaphore after drawing

* @note Selected menu item is highlighted with red (COLOR_OBSTACLE) text

*/

void drawMenu() {

// Take mutex to protect frame buffer access

xSemaphoreTake(bufferMutex, portMAX_DELAY);

// Clear screen to background color

Paint_Clear(COLOR_BG);

// Draw menu title

Paint_DrawString_EN(40, 30, "GAME MENU", &Font24, COLOR_FG, COLOR_BG);

// Draw menu options with selection highlighting

// Selected item is shown in red (COLOR_OBSTACLE), others in black (COLOR_FG)

Paint_DrawString_EN(30, 80, "1. Math Quiz", &Font20,

menuSelection == 0 ? COLOR_OBSTACLE : COLOR_FG, COLOR_BG);

Paint_DrawString_EN(30, 110, "2. Snake", &Font20,

menuSelection == 1 ? COLOR_OBSTACLE : COLOR_FG, COLOR_BG);

Paint_DrawString_EN(30, 140, "3. Power Off", &Font20,

menuSelection == 2 ? COLOR_OBSTACLE : COLOR_FG, COLOR_BG);

// Draw control instructions at bottom of screen

Paint_DrawString_EN(30, 180, "UP/DOWN - Select", &Font12, COLOR_FG, COLOR_BG);

Paint_DrawString_EN(30, 200, "A - Start Game", &Font12, COLOR_FG, COLOR_BG);

Paint_DrawString_EN(30, 220, "B - Back to Menu", &Font12, COLOR_FG, COLOR_BG);

// Release mutex and signal display task that frame is ready

xSemaphoreGive(bufferMutex);

xSemaphoreGive(frameSemaphore);

}

/* ================== FreeRTOS Tasks ================== */

/**

* @brief FreeRTOS task that reads button inputs with debouncing

*

* This task continuously polls all button GPIO pins and sends input state

* changes to the input queue. It implements debouncing by requiring a minimum

* time interval (DEBOUNCE_TIME) between state changes before sending updates.

*

* The task:

* - Reads all button states from GPIO pins

* - Compares current state with previous state

* - Only sends to queue if state changed and debounce time elapsed

* - Updates last known state and debounce timer

*

* @param pvParameters Task parameters (unused)

*

* @note Buttons are active-low (inverted logic: !DEV_Digital_Read)

* @note Debounce time is DEBOUNCE_TIME milliseconds (50ms default)

* @note Task runs at 10ms interval for responsive input

* @note Queue sends are non-blocking (timeout 0)

*/

void inputTask(void *pvParameters) {

InputState input = {0}, lastInput = {0};

TickType_t lastDebounce = 0;

// Infinite loop - task runs forever

while (1) {

TickType_t now = xTaskGetTickCount();

// Check if debounce time has elapsed since last state change

if (now - lastDebounce >= pdMS_TO_TICKS(DEBOUNCE_TIME)) {

// Read all button states (buttons are active-low, so invert)

input.btnA = !DEV_Digital_Read(BTN_A);

input.btnB = !DEV_Digital_Read(BTN_B);

input.btnUp = !DEV_Digital_Read(BTN_UP);

input.btnDown = !DEV_Digital_Read(BTN_DOWN);

input.btnLeft = !DEV_Digital_Read(BTN_LEFT);

input.btnRight = !DEV_Digital_Read(BTN_RIGHT);

input.btnCtrl = !DEV_Digital_Read(BTN_CTRL);

// Only send to queue if input state changed

if (memcmp(&input, &lastInput, sizeof(InputState)) != 0) {

// Send new input state to queue (non-blocking)

xQueueSend(inputQueue, &input, 0);

// Update last known state

lastInput = input;

// Reset debounce timer

lastDebounce = now;

}

}

// Delay before next button check (10ms = 100 checks per second)

vTaskDelay(pdMS_TO_TICKS(10));

}

}

/**

* @brief FreeRTOS task that handles game logic and rendering

*

* This task is the main game loop that:

* - Receives input events from the input queue

* - Handles global input (Button B returns to menu from any game)

* - Updates current game state based on game type

* - Renders current game screen

* - Maintains consistent frame timing using vTaskDelayUntil

*

* The task switches between different game modes:

* - GAME_MENU: Menu navigation and selection

* - GAME_MATH_QUIZ: Math quiz game logic and rendering

* - GAME_SNAKE: Snake game logic and rendering

*

* @param pvParameters Task parameters (unused)

*

* @note Uses vTaskDelayUntil for consistent frame timing

* @note Frame rate is 150ms per frame (~6.7 FPS) for menu and games

* @note Button B always returns to menu from any game

* @note Queue receives are non-blocking (timeout 0)

*/

void gameLogicTask(void *pvParameters) {

InputState input = {0};

TickType_t lastWakeTime = xTaskGetTickCount();

// Infinite loop - task runs forever

while (1) {

// Check for input events (non-blocking)

if (xQueueReceive(inputQueue, &input, 0) == pdTRUE) {

// Button B always returns to menu from any game

if (input.btnB && currentGame != GAME_MENU) {

currentGame = GAME_MENU;

}

}

// Update and render based on current game type

if (currentGame == GAME_MENU) {

updateMenu(&input);

drawMenu();

// Maintain 150ms frame time (~6.7 FPS)

vTaskDelayUntil(&lastWakeTime, pdMS_TO_TICKS(150));

} else if (currentGame == GAME_MATH_QUIZ) {

updateMathQuiz(&input);

drawMathQuiz();

// Maintain 150ms frame time (~6.7 FPS)

vTaskDelayUntil(&lastWakeTime, pdMS_TO_TICKS(150));

} else if (currentGame == GAME_SNAKE) {

updateSnake(&input);

drawSnake();

// Maintain 150ms frame time (~6.7 FPS)

vTaskDelayUntil(&lastWakeTime, pdMS_TO_TICKS(150));

}

}

}

/**

* @brief FreeRTOS task that handles LCD display updates

*

* This task waits for frame ready signals from game logic tasks and

* transfers the frame buffer to the LCD display. It uses semaphores

* to synchronize with game rendering:

* - Waits for frameSemaphore (signaled when new frame is ready)

* - Takes bufferMutex to safely access frame buffer

* - Transfers frame buffer to LCD display hardware

* - Releases mutex

*

* This decouples display timing from game logic, allowing games to

* render at their own pace while display updates happen asynchronously.

*

* @param pvParameters Task parameters (unused)

*

* @note Blocks on frameSemaphore until game logic signals new frame

* @note Uses mutex to protect frame buffer during LCD transfer

* @note Lower priority than game logic task to avoid blocking rendering

*/

void displayTask(void *pvParameters) {

// Infinite loop - task runs forever

while (1) {

// Wait for frame ready signal from game logic task

if (xSemaphoreTake(frameSemaphore, portMAX_DELAY) == pdTRUE) {

// Take mutex to protect frame buffer during LCD transfer

xSemaphoreTake(bufferMutex, portMAX_DELAY);

// Transfer frame buffer to LCD display hardware

LCD_1IN3_Display(frameBuffer);

// Release mutex

xSemaphoreGive(bufferMutex);

}

}

}

/* ================== Main Function ================== */

/**

* @brief Main function that initializes hardware and starts FreeRTOS scheduler

*

* This function performs the following initialization steps:

* 1. Initializes stdio for serial communication

* 2. Initializes Waveshare LCD display module and graphics buffer

* 3. Configures all button GPIO pins

* 4. Creates FreeRTOS queues and semaphores for inter-task communication

* 5. Creates and starts all FreeRTOS tasks

* 6. Starts the FreeRTOS scheduler

*

* Tasks created:

* - Input task (priority 3): Reads button inputs with debouncing

* - Game Logic task (priority 2): Handles game state and rendering

* - Display task (priority 1): Updates LCD display hardware

*

* @return int Exit status (never returns due to infinite scheduler loop)

*

* @note Returns -1 on initialization failure (display or memory allocation)

* @note Never returns under normal operation (scheduler runs forever)

*/

int main() {

// === 1) Initialize Standard I/O ===

// Initialize standard I/O (UART for debugging and serial communication)

stdio_init_all();

// Small delay to allow UART to stabilize

DEV_Delay_ms(100);

// === 2) Initialize Waveshare LCD Display Module ===

// Initialize the Waveshare device module (SPI, GPIO, etc.)

if (DEV_Module_Init() != 0) return -1;

// Set LCD backlight PWM to 70% brightness

DEV_SET_PWM(70);

// Initialize LCD display hardware in horizontal orientation

LCD_1IN3_Init(HORIZONTAL);

// Clear LCD to white

LCD_1IN3_Clear(WHITE);

// === 3) Allocate Display Frame Buffer ===

// Calculate frame buffer size: width * height * 2 bytes per pixel (RGB565)

UDOUBLE imageSize = LCD_1IN3_HEIGHT * LCD_1IN3_WIDTH * 2;

// Allocate memory for frame buffer

frameBuffer = (UWORD *)malloc(imageSize);

if (!frameBuffer) return -1;

// Initialize graphics library with frame buffer

Paint_NewImage((UBYTE *)frameBuffer, LCD_1IN3.WIDTH, LCD_1IN3.HEIGHT, 0, WHITE);

// Set font scale to 65

Paint_SetScale(65);

// Set rotation to 0 degrees (normal orientation)

Paint_SetRotate(ROTATE_0);

// === 4) Configure Button GPIO Pins ===

// Configure all button pins for input

DEV_KEY_Config(BTN_A);

DEV_KEY_Config(BTN_B);

DEV_KEY_Config(BTN_UP);

DEV_KEY_Config(BTN_DOWN);

DEV_KEY_Config(BTN_LEFT);

DEV_KEY_Config(BTN_RIGHT);

DEV_KEY_Config(BTN_CTRL);

// === 5) Create FreeRTOS Queues and Semaphores ===

// Create queue for input state messages (5 messages capacity)

inputQueue = xQueueCreate(5, sizeof(InputState));

// Create binary semaphore for frame ready signaling

frameSemaphore = xSemaphoreCreateBinary();

// Create mutex for frame buffer protection

bufferMutex = xSemaphoreCreateMutex();

// === 6) Create FreeRTOS Tasks ===

// Create input task: reads buttons, sends to queue (priority 3, stack 256 bytes)

xTaskCreate(inputTask, "Input", 256, NULL, 3, NULL);

// Create game logic task: updates games, renders frames (priority 2, stack 1024 bytes)

xTaskCreate(gameLogicTask, "Logic", 1024, NULL, 2, NULL);

// Create display task: updates LCD hardware (priority 1, stack 256 bytes)

xTaskCreate(displayTask, "Display", 256, NULL, 1, NULL);

// === 7) Start FreeRTOS Scheduler ===

// Start the FreeRTOS real-time scheduler (never returns)

vTaskStartScheduler();

// This code should never be reached

return 0;

}

Висновок: У ході лабораторної роботи було створено багатозадачну міні-консоль на FreeRTOS, де кожна частина системи жила у своєму власному ритмі: опитування кнопок, ігрова логіка та рендер кадрів. Вдалося реалізувати черги, семафори і м’ютекси, перетворивши хаос апаратних подій на кероване і пружне ігрове середовище. До консолі було додано меню, паузу та кілька повноцінних ігор, що реагують на користувача в реальному часі. Робота показала, як RTOS дозволяє створювати плавні, швидкі і чітко синхронізовані інтерактивні застосунки на мікроконтролерах.