Міністерство освіти і науки України

Львівський національний університет імені Івана Франка

Факультет електроніки та комп'ютерних технологій

ЗВІТ

про виконання лабораторної роботи № 8

з курсу “Операційні системи реального часу”

на тему: “Управління багатозадачною системою за допомогою ІЧ-пульта”

Виконав:

Студент. гр. ФеП-41 Фіняк Олег

Перевірив:

Павлик М.Р.

Львів – 2025

Мета роботи. Навчитися приймати та обробляти сигнали від інфрачервоного пульта дистанційного керування та реалізувати вибір задач або режимів роботи системи за допомогою ІЧ-пульта.

#include <stdio.h> #include <string.h> #include <stdbool.h>

/* Pico SDK */ #include "pico/stdlib.h"

#include "hardware/adc.h" #include "hardware/gpio.h" #include "hardware/pio.h"

/* FreeRTOS */ #include "FreeRTOS.h" #include "task.h" #include "queue.h" #include "semphr.h"

/* Waveshare Libs (OLED) */ #include "DEV_Config.h" #include "GUI_Paint.h" #include "OLED_1in5.h"

/* Sensor Libs */ #include "SHTC3.h" #include "SGP40.h" #include "QMI8658.h"

/* IR Receiver */ #include "nec_receive.h"

/* Defines */

#define PIO_NUM (pio0) #define IR_RX_PIN (15)

#define BUTTON_PIN 3

#define PIN_POT 26

#define PIN_LDR 27

#define PIN_MIC 28

/* RTOS Handles */

static SemaphoreHandle_t i2c_mutex; static QueueHandle_t xQueueButton; static QueueHandle_t xQueueSHTC3; static QueueHandle_t xQueueSGP40; static QueueHandle_t xQueueQMI8658; static QueueHandle_t xQueuePot;

static QueueHandle_t xQueueLight; static QueueHandle_t xQueueSound; static QueueHandle_t xQueueIR;

/* Global OLED Buffer */ static UBYTE *BlackImage;

/* Global PIO and State Machine for IR */ static PIO pio;

static int rx_sm;

/* Data Structures for Queues */ typedef struct {

float temp; float hum;

} SHTC3_data_t;

typedef struct { float acc[3]; float gyro[3];

} QMI8658_data_t;

typedef struct { uint8_t address; uint8_t data;

} IR_command_t;

typedef enum { SENSOR_SHTC3 = 0, SENSOR_SGP40, SENSOR_QMI8658, SENSOR_POT, SENSOR_LIGHT, SENSOR_SOUND, SENSOR_COUNT

} sensor_display_t;

/**

* @brief Task that polls button with debounce logic.

*/

void vButtonTask(void *pvParameters) { gpio_init(BUTTON_PIN); gpio_set_dir(BUTTON_PIN, GPIO_IN); gpio_pull_up(BUTTON_PIN);

bool last_state = true; // true = released (due to pull-up)

while (1) {

bool current_state = gpio_get(BUTTON_PIN);

// If button is pressed (false) and was previously released (true) if (current_state == false && last_state == true) {

// Debounce delay vTaskDelay(pdMS_TO_TICKS(50)); current_state = gpio_get(BUTTON_PIN);

// If still pressed, send event if (current_state == false) { bool pressed = true;

xQueueSend(xQueueButton, &pressed, 0);

}

}

last_state = current_state; vTaskDelay(pdMS_TO_TICKS(50));

}

}

/**

* @brief Task for reading temperature/humidity sensor SHTC3.

*/

void vSHTC3Task(void *pvParameters) { SHTC3_data_t data;

while (1) {

// Take mutex for I2C access

if (xSemaphoreTake(i2c_mutex, portMAX_DELAY) == pdTRUE) { SHTC3_Measurement(&data.temp, &data.hum); xSemaphoreGive(i2c_mutex);

// Send data to queue xQueueSend(xQueueSHTC3, &data, 0);

}

vTaskDelay(pdMS_TO_TICKS(100)); // Read every 100 ms

}

}

/**

* @brief Task for reading air quality sensor SGP40.

*/

void vSGP40Task(void *pvParameters) { uint32_t voc_index;

while (1) {

if (xSemaphoreTake(i2c_mutex, portMAX_DELAY) == pdTRUE) {

// Use T=25C, H=50% as in example voc_index = SGP40_MeasureVOC(25, 50); xSemaphoreGive(i2c_mutex);

xQueueSend(xQueueSGP40, &voc_index, 0);

}

vTaskDelay(pdMS_TO_TICKS(100)); // Read every 100 ms

}

}

/**

* @brief Task for reading accelerometer/gyroscope QMI8658.

*/

void vQMI8658Task(void *pvParameters) { QMI8658_data_t data;

unsigned int tim_count = 0;

while (1) {

if (xSemaphoreTake(i2c_mutex, portMAX_DELAY) == pdTRUE) { QMI8658_read_xyz(data.acc, data.gyro, &tim_count); xSemaphoreGive(i2c_mutex);

xQueueSend(xQueueQMI8658, &data, 0);

}

vTaskDelay(pdMS_TO_TICKS(100)); // Read every 100 ms

}

}

/**

* @brief Task for reading potentiometer from ADC channel 0 (GPIO26).

*/

void vPotTask(void *pvParameters) { while (1) {

adc_select_input(0); // GPIO26 -> ADC channel 0 sleep_us(5); // Allow multiplexer to settle uint16_t value = adc_read();

xQueueSend(xQueuePot, &value, 0);

vTaskDelay(pdMS_TO_TICKS(50)); // Read every 50 ms

}

}

/**

* @brief Task for reading photoresistor from ADC channel 1 (GPIO27).

*/

void vLightTask(void *pvParameters) { while (1) {

adc_select_input(1); // GPIO27 -> ADC channel 1 sleep_us(5); // Allow multiplexer to settle uint16_t value = adc_read();

xQueueSend(xQueueLight, &value, 0);

vTaskDelay(pdMS_TO_TICKS(50)); // Read every 50 ms

}

}

/**

* @brief Task for reading sound sensor from ADC channel 2 (GPIO28).

*/

void vSoundTask(void *pvParameters) { while (1) {

adc_select_input(2); // GPIO28 -> ADC channel 2 sleep_us(5); // Allow multiplexer to settle uint16_t value = adc_read();

xQueueSend(xQueueSound, &value, 0);

vTaskDelay(pdMS_TO_TICKS(50)); // Read every 50 ms

}

}

/**

* @brief Task for receiving and decoding IR remote signals.

*/

void vIRTask(void *pvParameters) { uint8_t rx_address, rx_data;

while (1) {

// Poll for IR frames

while (!pio_sm_is_rx_fifo_empty(pio, rx_sm)) { uint32_t rx_frame = pio_sm_get(pio, rx_sm);

if (nec_decode_frame(rx_frame, &rx_address, &rx_data)) {

// Only send commands for address 0x00 with data 0x07 or 0x15

if (rx_address == 0x00 && (rx_data == 0x07 || rx_data == 0x15)) { IR_command_t cmd;

cmd.address = rx_address; cmd.data = rx_data; xQueueSend(xQueueIR, &cmd, 0);

printf("IR command: %02x, %02x\r\n", rx_address, rx_data);

}

}

}

vTaskDelay(pdMS_TO_TICKS(10)); // Poll every 10 ms

}

}

/**

* @brief Task for displaying sensor data on OLED with sequential cycling.

*/

void vDisplayTask(void *pvParameters) {

static sensor_display_t current_sensor = SENSOR_SHTC3;

// Local variables for storing latest data SHTC3_data_t shtc3_data = {0}; uint32_t voc_data = 0;

QMI8658_data_t qmi_data = {{0}, {0}}; uint16_t pot_data = 0;

uint16_t light_data = 0; uint16_t sound_data = 0; IR_command_t ir_cmd; char str[40];

while (1) {

// Check for IR commands (non-blocking)

if (xQueueReceive(xQueueIR, &ir_cmd, 0) == pdPASS) { if (ir_cmd.address == 0x00) {

if (ir_cmd.data == 0x07) {

// Minus button: cycle to previous sensor

current_sensor = (sensor_display_t)((current_sensor - 1 + SENSOR_COUNT) % SENSOR_COUNT);

} else if (ir_cmd.data == 0x15) {

// Plus button: cycle to next sensor

current_sensor = (sensor_display_t)((current_sensor + 1) % SENSOR_COUNT);

}

}

}

// Receive data from all sensor queues (non-blocking) xQueueReceive(xQueueSHTC3, &shtc3_data, 0);

xQueueReceive(xQueueSGP40, &voc_data, 0);

xQueueReceive(xQueueQMI8658, &qmi_data, 0);

xQueueReceive(xQueuePot, &pot_data, 0);

xQueueReceive(xQueueLight, &light_data, 0);

xQueueReceive(xQueueSound, &sound_data, 0);

// Display current sensor data Paint_SelectImage(BlackImage); Paint_Clear(BLACK);

switch (current_sensor) { case SENSOR_SHTC3:

sprintf(str, "SHTC3");

Paint_DrawString_EN(5, 5, str, &Font12, WHITE, BLACK); sprintf(str, "T: %.2f C", shtc3_data.temp); Paint_DrawString_EN(5, 20, str, &Font12, WHITE, BLACK); sprintf(str, "H: %.2f %%", shtc3_data.hum); Paint_DrawString_EN(5, 35, str, &Font12, WHITE, BLACK); break;

case SENSOR_SGP40:

sprintf(str, "SGP40");

Paint_DrawString_EN(5, 5, str, &Font12, WHITE, BLACK); sprintf(str, "VOC: %u", (unsigned int)voc_data); Paint_DrawString_EN(5, 20, str, &Font12, WHITE, BLACK);

break;

case SENSOR_QMI8658:

sprintf(str, "QMI8658");

Paint_DrawString_EN(5, 5, str, &Font12, WHITE, BLACK); sprintf(str, "AccX: %.2f", qmi_data.acc[0]); Paint_DrawString_EN(5, 20, str, &Font12, WHITE, BLACK); sprintf(str, "AccY: %.2f", qmi_data.acc[1]); Paint_DrawString_EN(5, 35, str, &Font12, WHITE, BLACK); sprintf(str, "AccZ: %.2f", qmi_data.acc[2]); Paint_DrawString_EN(5, 50, str, &Font12, WHITE, BLACK); sprintf(str, "GyroX: %.2f", qmi_data.gyro[0]); Paint_DrawString_EN(5, 65, str, &Font12, WHITE, BLACK); sprintf(str, "GyroY: %.2f", qmi_data.gyro[1]); Paint_DrawString_EN(5, 80, str, &Font12, WHITE, BLACK); sprintf(str, "GyroZ: %.2f", qmi_data.gyro[2]); Paint_DrawString_EN(5, 95, str, &Font12, WHITE, BLACK); break;

case SENSOR_POT:

sprintf(str, "Potentiometer");

Paint_DrawString_EN(5, 5, str, &Font12, WHITE, BLACK); sprintf(str, "Value: %u", pot_data);

Paint_DrawString_EN(5, 20, str, &Font12, WHITE, BLACK); sprintf(str, "Percent: %.1f%%", (pot_data * 100.0f / 4095.0f)); Paint_DrawString_EN(5, 35, str, &Font12, WHITE, BLACK); break;

case SENSOR_LIGHT:

sprintf(str, "Photoresistor");

Paint_DrawString_EN(5, 5, str, &Font12, WHITE, BLACK); sprintf(str, "Value: %u", light_data);

Paint_DrawString_EN(5, 20, str, &Font12, WHITE, BLACK); sprintf(str, "Percent: %.1f%%", (light_data * 100.0f / 4095.0f)); Paint_DrawString_EN(5, 35, str, &Font12, WHITE, BLACK); break;

case SENSOR_SOUND:

sprintf(str, "Sound Sensor");

Paint_DrawString_EN(5, 5, str, &Font12, WHITE, BLACK); sprintf(str, "Value: %u", sound_data); Paint_DrawString_EN(5, 20, str, &Font12, WHITE, BLACK);

sprintf(str, "Percent: %.1f%%", (sound_data * 100.0f / 4095.0f)); Paint_DrawString_EN(5, 35, str, &Font12, WHITE, BLACK); break;

default: break;

}

OLED_1in5_Display(BlackImage);

// Update display every 200 ms vTaskDelay(pdMS_TO_TICKS(200));

}

}

int main(void) {

// Initialize system stdio_init_all();

if (DEV_Module_Init() != 0) { printf("DEV_Module_Init failed\r\n"); while(1);

}

printf("DEV_Module_Init OK\r\n");

// Initialize OLED OLED_1in5_Init(); OLED_1in5_Clear(0);

// Create display buffer

UWORD Imagesize = ((OLED_1in5_WIDTH % 2 == 0) ? (OLED_1in5_WIDTH / 2) : (OLED_1in5_WIDTH / 2

+ 1)) * OLED_1in5_HEIGHT;

if ((BlackImage = (UBYTE *)malloc(Imagesize)) == NULL) { printf("Failed to apply for black memory...\r\n");

while(1);

}

Paint_NewImage(BlackImage, OLED_1in5_WIDTH, OLED_1in5_HEIGHT, 0, WHITE); Paint_SetScale(16);

Paint_Clear(BLACK); OLED_1in5_Display(BlackImage);

// Initialize PIO for IR receiver pio = PIO_NUM;

rx_sm = nec_rx_init(pio, IR_RX_PIN); if (rx_sm == -1) {

printf("could not configure PIO\n"); while(1);

}

printf("IR RX initialized\r\n");

// Initialize ADC hardware adc_init();

adc_gpio_init(PIN_POT); // GPIO26 -> ADC channel 0 adc_gpio_init(PIN_LDR); // GPIO27 -> ADC channel 1 adc_gpio_init(PIN_MIC); // GPIO28 -> ADC channel 2 printf("ADC initialized\r\n");

// Initialize I2C mutex

i2c_mutex = xSemaphoreCreateMutex(); if (i2c_mutex == NULL) {

printf("Failed to create I2C mutex\r\n"); while(1);

}

// Initialize I2C sensors (with mutex protection) xSemaphoreTake(i2c_mutex, portMAX_DELAY); SHTC3_Init();

SGP40_init(); QMI8658_init(); xSemaphoreGive(i2c_mutex); printf("Sensors Init OK\r\n");

// Create queues

xQueueButton = xQueueCreate(1, sizeof(bool)); xQueueSHTC3 = xQueueCreate(1, sizeof(SHTC3_data_t)); xQueueSGP40 = xQueueCreate(1, sizeof(uint32_t)); xQueueQMI8658 = xQueueCreate(1, sizeof(QMI8658_data_t)); xQueuePot = xQueueCreate(1, sizeof(uint16_t));

xQueueLight = xQueueCreate(1, sizeof(uint16_t));

xQueueSound = xQueueCreate(1, sizeof(uint16_t)); xQueueIR = xQueueCreate(4, sizeof(IR_command_t));

// Check if all queues were created successfully

if (!xQueueButton || !xQueueSHTC3 || !xQueueSGP40 || !xQueueQMI8658 ||

!xQueuePot || !xQueueLight || !xQueueSound || !xQueueIR) { printf("Failed to create queues\r\n");

while(1);

}

// Create tasks

xTaskCreate(vButtonTask, "ButtonTask", 256, NULL, 1, NULL);

xTaskCreate(vSHTC3Task, "SHTC3Task", 256, NULL, 1, NULL);

xTaskCreate(vSGP40Task, "SGP40Task", 256, NULL, 1, NULL);

xTaskCreate(vQMI8658Task, "QMI8658Task", 256, NULL, 1, NULL);

xTaskCreate(vPotTask, "PotTask", 256, NULL, 1, NULL);

xTaskCreate(vLightTask, "LightTask", 256, NULL, 1, NULL);

xTaskCreate(vSoundTask, "SoundTask", 256, NULL, 1, NULL);

xTaskCreate(vIRTask, "IRTask", 256, NULL, 3, NULL);

xTaskCreate(vDisplayTask, "DisplayTask", 512, NULL, 1, NULL);

printf("All tasks created, starting scheduler\r\n");

// Start scheduler vTaskStartScheduler();

// This code should never be reached while (1) {

// NOP

}

Висновок: У роботі було створено повноцінну багатозадачну систему FreeRTOS із підтримкою ІЧ-пульта за протоколом NEC, сенсорів, аналогових датчиків та OLED-дисплея. Дані від усіх задач передавалися через черги, а доступ до I²C захищався м’ютексом, що забезпечує коректну взаємодію сенсорів. Дисплей отримував дані з різних джерел та перемикав режими роботи відповідно до команд пульта. Система працює в реальному часі та адекватно реагує на дії користувача, забезпечуючи стабільне та передбачуване керування.